# PAL-1 Motherboard Expansion Kit

This is an unassembled kit and requires soldering.  All components are through-hole.

With this motherboard, you can connect up to 6 expansion boards to your PAL-1 at the same time.  The motherboard also provides the Application and Expansion connectors found on the original KIM-1, enabling the use of original KIM-1 expansion boards.

NOTE: To do anything worthwhile with the Application connector you will need the PAL-1 2nd RIOT Expansion Kit.

Not all signals are routed to the Application and Expansion connectors because the PAL-1 doesn’t provide them, so there is no guarantee every expansion board will work.  However, Corsham Technologies’ and RetroSpy Technologies KIM-1 I/O and KIM-1 RAM/ROM boards have been tested and are compatible.

Available signals on the Application connector:

- +5V

- GND

- PA0-7 (with installed PAL-1 2nd RIOT Expansion Kit)

- PB0-7 (with installed PAL-1 2nd RIOT Expansion Kit)

- DECODE ENAB

Available signals on the Expansion connector:

- +5V

- GND

- A0-A15

- D0-D7

- SYNC

- RDY

- IRQ

- NMI

- RST

- SST OUT

- PHI2

- R/W

- /R/W

- /PHI2

- RAM/R/W

## Assembly

Assembly is pretty straightforward.  The only thing to note is that the notch on the 2x9 box header should face the Application connector.

## Usage

  - Attach the 2x9 ribbon cable between the motherboard and the installed PAL-1 2nd RIOT Expansion Kit.

  - Attach the 2x20 ribbon cable between the motherboard and the PAL-1

  - Power on the PAL-1

## Notes on using the RetroSpy Technologies I/O and RAM/ROM Boards

- Ensure both the PAL-1’s DECODE jumper and the I/O boards jumper are both set to EXT if using the RAM/ROM board

- The K1-4 DIP switches on the I/O board should be off, since the PAL-1 already provides this extra 4K of RAM.

- TTY<->KBD switch on the I/O board does do anything because it is not connected to the PAL-1.

- It might work, but I wouldn’t recommend having a 5v supply plugged into the I/O board

- A 12v supply on the I/O board can be safely used to power the SD Card System, since +12v isn’t passed to the PAL-1

- AUDIO IN and AUDIO OUT on the I/O board will not work as those signals are not passed from the PAL-1.

- The serial connection on the I/O board will not work as the PAL-1 has its own serial connection.